ActiveX Userfunctions and Userobjects used for VEE/MS Access 97

## UFSendRecordArrayToDatabase

Function: Appends inputted record array to currently open recordset.

Input: Record array to be appended to database.

Assumes: A connection already exists to the database and a recordset called rsqryresults is open (cannot be read only – I usually use the parameter adLockOptimistic).  There must be a preexisting field name in the database that corresponds to each field name in the record array.

How it works: The function determines the total number of records in the record array and sets up a For-Count loop.  Each iteration of the loop sends the next record in the record array to the function UFSendToDatabase until all records in the record array have been sent to the database.

Calls these UFs: UFSendToDatabase, ErrorReport (error handler)

## Where used: TurboCal.vee

## UFSendToDatabase

Function: Appends inputted record scalar to currently open recordset.

Input: Record scalar to be appended to database.

Assumes: A connection already exists to the database and a recordset called rsqryresults is open (cannot be read only – I usually use the parameter adLockOptimistic).  There must be a preexisting field name in the database that corresponds to each field name in the record array.

How it works: The function starts by creating a new blank record in the database using the Addnew method.  Then it cycles through each field of the record scalar, placing the contents of the field in the corresponding field of the database.

Calls these UFs: ErrorReport (error handler)

Where used: TurboQC.vee, SpecificCal.vee, TurboCal.vee

## UFConvertRecordsetToRecord

Function: Converts inputted recordset to a record array (or record scalar if there is only one record) and places record array on its output pin.

Input: Recordset to be converted, record scalar containing field names of desired fields to be extracted from the recordset.  The values contained in this record scalar don’t matter – they are not used or manipulated.

Assumes: A recordset is open.  It does not matter if a connection is open.  (If the connection is closed prior to the recordset being sent to this function, the property ActiveConnection of the recordset must be set to Nothing (e.g. cn.ActiveConnection = Nothing).  There must be a preexisting field name in the database that corresponds to each field name in the record scalar.

How it works: The recordset cursor is positioned at the top record, (MoveFirst) then each field is extracted by 'rsqryresults(field)' and stored in the tempRec.  For each extracted record, tempRec is added to the 'collector' to build an array of records mirroring the database table.  When the table is at End Of File (EOF), the collector is fired and outputs its VEE record array.

An error will occur if the userfunction attempts to access a field that does not exist.  However, not ALL fields have to be extracted.  If only a few fields are desired, simply input a record scalar with the desired fields.

Calls these UFs: ErrorReport (error handler)

Where used: TurboQC.vee, KelvinKlamp, SpecificCal.vee

## UFEditRecord

Function: Updates current record of recordset rsqryresults with values of record scalar inputed into function.

Input: Record scalar with new values.

Assumes: A recordset named rsqryresults has been opened and the current record is the one that should be edited.

Calls these UFs: ErrorReport (error handler)

Where used: TurboCal.vee, KelvinKlamp.vee

## UFRetrieveTable

Function: Places entire contents of specified table or query into a record scalar (1 record) or record array (more than one record).

Input: Name of table or query to retrieve.

Assumes: A separate userfunction exists that builds a “dummy” record containing the field names you want to retrieve from the table or query.  This should be named “usrfBuildTableNameRecord”, where TableName is the name of the table or query. Requires a variable called DatabaseFilePath to be set that contains the path to the database.  (i.e. "E:\\temp\\275_gauge.mdb")

Calls these UFs: ErrorReport, UFBuildTableNameRecord, UFConvertRecordsetToRecord

Where used: TurboCal.vee

## UFAppendTable

Function: Appends contents of inputed record array into specified table.

Input: Record array of any size; name of Access table to be appended to.

Assumes: Two tables already exist in Access with the name XXX and XXXTemp, where XXX is the name of the table inputed into the userfunction.  The structure of the two tables should be the same.  The field names of the record must also match the field names in Access.  The record does not have to have every field name that’s contained in the table.  If you only want to update 10 of the 30 fields in the table, you can use a record with only 10 fields.  The rest will be left empty.  Requires a variable called DatabaseFilePath to be set that contains the path to the database.  (i.e. "E:\\temp\\275_gauge.mdb")

Note by RDM 3/13/00: This function has been changed slightly to create a temp table based on the value in the Station variable.  For example, if the inputted table name is ‘tblCal’ and the Station variable contains ‘3’, then the temporary table name will be ‘tblCal3Temp’.

How it works: The user calls this function and sends it two variables: the record array containing the data and the name of the table to append the data to.  The userfunction starts by deleting all records from the XXXTemp table (e.g. tblCal3Temp).  (In reality, the table should already be empty but this step is done just to make sure.)  It then populates this table with the contents of the record.  Then it appends all records in this table to the main table (e.g. tblCal3 – whatever name the user sent to the userfunction).  The last step is to delete all records from the table and close the connection.

Calls these UFs: ErrorReport, UFSendRecordArrayToDatabase, UFSendToDatabase

Where used: TurboCal.vee

## UFBuildTable

Function: Deletes contents of specified table, then inserts contents of inputed record into this table.  Thus entire contents of table is guaranteed to be only what has been inputed.

Input: Record array of any size; name of Access table to be updated.

Assumes: A table already exists in Access with the specified name.  The field names of the record must match the field names in Access.  The record does not have to have every field name that’s contained in the table.  If you only want to update 10 of the 30 fields in the table, you can use a record with only 10 fields.  The rest will be left empty.  Requires a variable called DatabaseFilePath to be set that contains the path to the database.  (i.e. "E:\\temp\\275_gauge.mdb")

Calls these UFs: ErrorReport, UFSendRecordArrayToDatabase, UFSendToDatabase

Where used: NOT USED

## UOUpdateDatabase

Function: Updates data in 275 calibration database with Kelvin Klamp data (R8, R8_real, etc.).  This object is called after a batch of 10 tubes has been Kelvin Klamped.

Input: None

Assumes: A variable named Batch_num is set that contains the batch number.  The data is stored in the table tblCal in the database.  Path to database is set in DatabaseFilePath.  Ten-element record array named rc exists that contains calibration data.

How it works: It starts by retrieving all records from the database that match the batch number.  This should be 10.  Then it uses a For-Count loop to loop through each record of the rc record array, extracting the desired fields, building a new record based on the desired fields, and sending that record scalar to the userfunction UFEditRecord.  This repeats until the For-Count is done – 10 times.

Calls these UFs: UFEditRecord, ErrorReport

Where used: KelvinKlamp.vee

## UOPrintQkReport

Function: Prints a Quick Report named rptQuickReportX, where X is the calibration station identifier, which is created based upon searching the table tblCal for the inputed batch number.

Input: The batch number for the desired report.  Hardcoded to look only in tblCal.

Assumes: Reports named rptQuickReport3, rptQuickReport4, rptQuickReport5, and rptQuickReportSub already exists in Access. (rptQuickReportSub is a subreport of the other reports.)  The default printer is the desired printer.  Requires a variable called DatabaseFilePath to be set that contains the path to the database (i.e. “\\\\mnt5\\My documents\\Database\\275_gauge.mdb") and a variable called Batch that contains the desired batch number (i.e. 31876).  A variable Station must be set that stores the station number (3, 4 or 5).  Access must be installed on the computer.

How it works: It starts by attempting to delete one of the tables named tblTemp3, tblTemp4, or tblTemp5 to ensure it doesn't exist.  Which table it tries to delete is determined by the value stored in the variable Station (3, 4 or 5).  Then it builds a new table named tblTemp3, tblTemp4, or tblTemp5 (depending on Station) that is populated with all records that match the inputed batch number.  This should be 10.  Then the report is opened and printed to the default printer.  The report is based on a qry that selects all records in tblTempX.  The report also contains the subreport rptQuickReportSub that displays the status numbers and their corresponding descriptions.  Finally, tblTempX is deleted from the database.

Calls these UFs: None

Where used: TurboCal.vee

## UOReset

Function: Attempts to gracefully exit any open instances of Access and delete the table tblTemp2.

Input:  References the global variable db, which represents an open database object. Also references a variable called DatabaseFilePath that contains the path to the database (i.e. "E:\\temp\\275_gauge.mdb").

Assumes: A database connection is already open and is represented by the variable db.

How it works: It attempts to carry out these actions, starting with the first one:

- Close the report with the name contained in the variable ReportName.

- Delete the table tblTemp2.

- Close the current database.

- Quit Microsoft Access.

If an error occurs during any of these actions, the program will automatically resume with the next one.  This prevents the program from crashing even if the user independently closes Access, while insuring the table tblTemp2 is properly deleted.

Calls these UFs: None

Where used: KelvinKlamp.vee