## CRC

Calculate Cyclic Redundancy Check

### Format

opcode tbl.ab, inicrc.rl, strlen.rw, stream.ab

### Condition Codes

| N¦¦ <--- R0 LSS 0; |  |
| --- | --- |
| Z¦¦ <--- R0 EQL 0; |  |
| V¦¦ <--- 0; |  |
| C¦¦ <--- 0; |  |

#### Exceptions

None.

### Opcodes

| 0B | CRC | Calculate Cyclic Redundancy Check |
| --- | --- | --- |

### Description

The CRC of the data stream described by the string descriptor is calculated. The initial CRC is given by inicrc; it is normally zero or -1, unless the CRC is calculated in several steps. The result is left in R0. If the polynomial is less than order 32, the result must be extracted from the low-order bits of R0. The CRC polynomial is expressed by the contents of the 16-longword table. See the notes for the calculation of the table.

Notes

- After execution:

| R0 = | Result of CRC |
| --- | --- |
| R1 = | 0 |
| R2 = | 0 |
| R3 = | Address 1 byte beyond the end of the source string |

- If the data stream is not a multiple of 8 bits, it must be right-adjusted with leading zero fill.

- If the CRC polynomial is less than order 32, the result must be extracted from the low-order bits of R0.

- Use the following algorithm to calculate the CRC table given a polynomial expressed:

polyn<n> <- {coefficient of x**{order -1-n}}

- The following routine is system library routine LIB$CRC_TABLE (poly.r1, table.ab). The table is the location of the 64-byte (16-longword) table into which the result will be written.

SUBROUTINE LIB$CRC_TABLE (POLY, TABLE) INTEGER*4 POLY, TABLE(0:15), TMP, X DO 190 INDEX = 0, 15 TMP = INDEX DO 150 I = 1, 4 X = TMP .AND. 1 TMP = ISHFT(TMP,-1) !logical shift right one bit IF (X .EQ. 1) TMP = TMP .XOR. POLY 150 CONTINUE TABLE(INDEX) = TMP 190 CONTINUE RETURN END

- The following are descriptions of some commonly used CRC polynomials:

CRC-16 (used in DDCMP and Bisync) polynomial: x^16 + x^15 + x^2 + 1 poly: 120001 (octal) initialize: 0 result: R0<15:0> CCITT (used in ADCCP, HDLC, SDLC) polynomial: x^16 + x^12 + x^5 + 1 poly: 102010 (octal) initialize: -1<15:0> result: one's complement of R0<15:0> AUTODIN-II polynomial: x^32+x^26+x^23+x^22+x^16+x^12 +x^11+x^10+x^8+x^7+x^5+x^4+x^2+x+1 poly: EDB88320 (hex) initialize: -1<31:0> result: one's complement of R0<31:0>

- The CRC instruction produces an UNPREDICTABLE result unless the table is well-formed, like the one produced in note 3. Note that for any well-formed table, entry[0] is always zero and entry[8] is always the polynomial expressed as in note 3. The operation can be implemented using shifts of 1, 2, or 4 bits at a time, as follows:

| Shift (s) | Steps per Byte (limit) | Table Index | Table Index Multiplier (i) | Use Table Entries |
| --- | --- | --- | --- | --- |
| 1 | 8 | tmp3<0> | 8 | [0]=0,[8] |
| 2 | 4 | tmp3<1:0> | 4 | [0]=0,[4],[8],[12] |
| 4 | 2 | tmp3<3:0> | 1 | all |

- If the stream has zero length, R0 receives the initial CRC.