File: REMOTE.DOC                                                 Oct 91

HP 11759C/D Remote Control Documentation

::::::::::::::

:: OVERVIEW ::

::::::::::::::

This document describes how to set up and use the remote control capabilities

of the Channel Simulator software.  The Channel Simulator can be controlled

remotely by either its serial I/O port or by an HP-IB bus.

Most menus can be accessed and controlled from a remote terminal or a

remote computer, such as a HP 9836A running BASIC, or equivalent.

No special HP-IB installation is needed.  Follow the directions in the

HP-IB card manual, the 11759 manual and the remote controller.

An example HP-IB program is included at the end of this document.

As an example, documentation for connecting and using an HP 9836A computer to

control a Vectra PC is provided.  Directions are included to configure the HP

9836A's serial datacom card (HP 98628A), and how to cable it to the PC.  An

example program is supplied, in HP BASIC, to download a complete test setup to

the Channel Simulator.  Configuration info is also provided for using the HP

Terminal Emulator product for the HP 9836A to control the Channel Simulator

remotely.

This serial remote control feature uses command lines to perform specific

tasks.  MENU TRAVEL, for example, might be sent from the remote controller to

the PC over this serial link to switch from the main menu to the TRAVEL test.

These word commands are well suited for use with an ATE (Automated Test

Equipment) controller that controls the Channel Simulator along with other ATE

hardware in one coordinated test system.  These commands allow error reporting

back to the ATE controller after each command, and they provide a way to hold

off the next command until the previous command has been executed by the

Channel Simulator.

In addition, single-key remote commands can be used to perform certain

auxiliary tasks.  Since these single-key commands do not return any status or

error messages, they are not as useful as the word commands for remote

control.  Accordingly, the above mentioned word-commands are the preferred

remote control technique for the Channel Simulator.

::::::::::::::

:: CONTENTS ::

::::::::::::::

1  OVERVIEW........................... This overview

1  INSTALLATION....................... How to configure hw & sw

2    --Hardware

2       o Cabling..................... How to cable between PC & Controller

4       o Interface Card.............. Switch & configure the HP 98628A Card

--Software Config

4       o ATE Controller-- HP 9836A .. Using BASIC or terminal emulator pgm.

5       o Sample RMB control program.. Sample program to control the HP 11759

10       o PC's Channel Simulator...... How to configure the software, trying it

11  REMOTE COMMANDS

11    -- Introduction.................. Overview of the commands.

11    -- Syntax Rules.................. Language rules and error handling

13    -- Error Handling................ How error conditions are handled.

13    -- Summary of Commands........... List of the remote control commands.

13         Main Menu

14         Simulation Test

16         Travel Test

17    -- Error Messages................ Errors, and what they mean.

26    -- One-key Remote Commands....... Summary of the one-key remote cmds.

27    -- Sample RMB HP-IB program...... Sample program to control the HP 11759

HP 11759C/D Remote Control Documentation

- 2 -

::::::::::::::::::::::::::::::

:: INSTALLATION -- HARDWARE ::

::::::::::::::::::::::::::::::

These directions assume the user has an HP-Vectra type serial interface

card in their PC, although these directions are probably similar for

other PC's.  Similarly, it's assumed that the PC is to be connected to a

HP 9836A (or similar S200, S300 computer) via the HP 98628A serial interface

card.  This interface card plugs into the backplane of S200 and S300

computers.  If a different controller (or terminal) is desired, the

wiring below should be compared to the wiring for your computer.

::::::::::::::::::::::::::::::::::::::

:: INSTALLATION -- HARDWARE Cabling ::

::::::::::::::::::::::::::::::::::::::

The easiest way to connect this Vectra PC to the HP 98628A interface card

on the S200 is with two premade cables.  The HP 24542M connects the

PC serial port to a 25-pin RS232 female connector.  The HP 98628A opt002

interface comes with cable HP PN 5061-4216, which has a 25-pin DB-25

connector on one end, and a 50-pin ribbon for connecting the interface

card.  A pictorial view of this connection is show as part of the wiring

schematic below.

The following example illustrates how to read this pictorial diagram.

Referring to the first connection in the diagram, this example verbally

traces out the top connection from left to right.

On the female 50-pin ribbon on the input to the HP 98628A interface card,

pin 13 is RTS, which outputs toward the PC, and pin 44 is CTS, which

reads back.  They are both joined together in the HP 98628A opt 002 cable

and then routed to pin 8 on the female DB25 side of the HP 98628A opt 002

cable.  That connection goes to pin 8 on the HP 24542M cable with the

25-pin male DB25 connector.  It gets routed in the HP 24542M cable to the

9-pin DB25 female connector on pin 1.  That connector is plugged into

the Vectra PC's 9-pin female, where pin 1 is routed to the CD input on

the Vectra.

The schematic is shown on the next page, and the discussion resumes on

page 4.

HP 11759C/D Remote Control Documentation

- 3 -

SCHEMATIC DIAGRAM OF CABLING--

< HP 98628A Card >     <5061-4216 Cable>      <HP 24542M Cable>  < HP Vectra >

HP 98628A opt 002                              PC

(50)                      (25)                (9)

F  M                      F  M               F   M

RTS-------->-(13)=-=-*-=-=-=-=-=-=-=-=-(8):::::::::::::::::(1)------->-CD

|

CTS-<--------(44)=-=-*

DATA OUT--->-(12)=-=-=-=-=-=-=-=-=-=-=-(3):::::::::::::::::(2)------->-Rx

DATA IN-<----(42)=-=-=-=-=-=-=-=-=-=-=-(2):::::::::::::::::(3)-<-------Tx

RI -<--------( 9)=-=-*-=-=-=-=-=-=-=-=(20):::::::::::::::::(4)-<-------DTR

|

DSR -<-------(45)=-=-*

GROUND -<->--( 7)=-=-=-=-=-=-=-=-=-=-=-(7):::::::::::::::::(5)--<--->-GROUND

DTR-------->-(14)=-=-*-=-=-=-=-=-=-=-=-(6):::::::::::::::::(6)------->-DSR

|

*-=-=-=-=-=-=-=-=(22)::::::<n.c.>

DCD -<-------(46)=-=-*-=-=-=-=-=-*-=-=-(4):::::::::::::::::(7)-<-------RTS

|

*-=-=-(5):::::::::::::::::(8)------->-CTS

<n.c>::::::::(9)------->-RI

CHASSIS GND--(24)=-=-=-=-=-=-=-=-=-=-=-(1):::::::::::::::(CASE)-<---->-C.GND

2nd DCD-<----(47)=-=-=-=-=-=-=-=-=-=-=(19)::::<n.c.>

2nd RTS---->-(15)=-=-=-=-=-=-=-=-=-=-=(12)::::<n.c.>

DCE Rec Tim<-(43)=-=-=-=-=-*-=-=-=-=-=(15)::::<n.c.>

|

DCE XmitTim->( 7)=-=-=-=-=-*-=-=-=-=-=(17)::::<n.c.>

Legend:  '-=-=-='  ==> HP 98628A OPT001 Cable 50-pin ribbon to 25pin Female

'::::::'  ==> HP 24542M Cable, 25-pinMale to 9-pin Female

'------'  ==> Interconnections within assemblies.

'  ( ) '  ==> Pin numbers at connectors.

'M', 'F'  ==> Male or female designation,or type of conn.

Note:  Some of the above lines can be omitted, if convenient.  The only

necessary connections are (using the Vectra PC Serial connector

pin numbering):

o Jumper pins 7 & 8 on the Vectra PC serial connector.

o Vectra Pin 6 (DSR) must be held above +5v by remote controller.

o Vectra Pins 2,3, & 5 are Vectra's transmit, receive, & ground.

HP 11759C/D Remote Control Documentation

- 4 -

SCHEMATIC DIAGRAM OF WIRING TO A S300 COMPUTER (MODEL 350 OR EQUIVALENT)

Note:  This setup has been researched, but not tested.

< Built-In RS232 >   < ------------ Custom Cable ----------->   < HP Vectra >

98651-66530                                                        PC

(9)                                           (9)

F  M                                         F   M

RTS-------->-( 4)::::::::::::::::::::::::::::::::::::::::::(1)------->-CD

::

CTS-<--------( 5)::::::

Tx -------->-( 2)::::::::::::::::::::::::::::::::::::::::::(2)------->-Rx

Rx -----<----( 3)::::::::::::::::::::::::::::::::::::::::::(3)-<-------Tx

RI -<--------( 9)::::::::::::::::::::::::::::::::::::::::::(4)-<-------DTR

::

DSR -<-------( 6)::::::

GROUND -<->--( 7)::::::::::::::::::::::::::::::::::::::::::(5)--<--->-GROUND

DTR-------->-( 1)::::::::::::::::::::::::::::::::::::::::::(6)------->-DSR

DCD -<-------( 8)::::::::::::::::::::::::::::::::::::::::::(7)-<-------RTS

::

::::::::::::::::::::::::::(8)------->-CTS

<n.c>::::::::(9)------->-RI

CHASSIS GND--(xx)::::::::::::::::::::::::::::::::::::::::(CASE)-<---->-C.GND

Legend:  '::::::'  ==> Custom-made 9-pin male to 9-pin male RS232 cable.

'------'  ==> Interconnections within assemblies.

'  ( ) '  ==> Pin numbers at connectors.

'M', 'F'  ==> Male or female designation,or type of conn.

Note:  Some of the above lines can be omitted, if convenient.  The only

necessary connections are (using the Vectra PC Serial connector

pin numbering):

o Jumper pins 7 & 8 on the Vectra PC serial connector.

o Vectra Pin 6 (DSR) must be held above +5v by remote controller.

o Vectra Pins 2,3, & 5 are Vectra's transmit, receive, & ground.

HP 11759C/D Remote Control Documentation

- 5 -

:::::::::::::::::::::::::::::::::::::::::::::

:: INSTALLATION -- HARDWARE Interface Card ::

:::::::::::::::::::::::::::::::::::::::::::::

The HP 98628A interface card has some switches that define the powerup state

for the various datacom parameters.  This section describes how to configure

them.  These parameters need to be set on whichever controller is being

used, whether that config is determined in hardware or software.

Make sure that the handshake line in the HP 98628A card is set to

HANDSHAKE OFF, NON-MODEM CONNECTION.  (This is a binary '00' on the 2nd

dip switch from the edge, on the 3rd & fourth bit toggles from that switch's

left side.

All the switches for the HP 98628A card could be set up as follows.  Since the

settings are programmable, however, the only one that needs to be set up is the

handshake line mentioned above.

|					Switch A:

| (part of HP 98628A card)		  b7:    (1)     ...R, currently unused

|					  b6,b5  (00)    ...Interupt level 3

|    b7     b0   b7      b0		  b4..b0 (10100) ...Select Code 20

|1   ________     ________

|   |switch A|   |switch B|		Switch B:

|0  |________|   |________|		  b7,b6  (01)    ...No Parity/7 Databits

|_________________________________      b5,b4  (00)    ...HS off;non-modem con

b3..b1 (010)   ...300 Baud, 1 stopbit

Fig 1. Pictorial of datacom card.	  b0     (0)     ...ASYNC data mode.

::::::::::::::::::::::::::::::::::::::::::::::::::::::::

:: INSTALLATION -- SOFTWARE ATE Controller-- HP 9836A ::

::::::::::::::::::::::::::::::::::::::::::::::::::::::::

It is strongly recommended that the serial data rate be kept at or below

300 baud, so the Channel Simulator software can keep up with the

controller's data rate.

A terminal emulator program could be run on the HP 9836A to control the

channel simulator.  The HP 98791B terminal emulator runs under the

Pascal Workstation on a S200 computer like the HP 9836A, or a S300 computer.

The terminal emulator is configured with the following parameters.  (See

the terminal emulator manual for instructions on how to enter these

parameters.)

DATACOM:

300 Baud     No Parity     No Check Parity      7 bits/char     1 stop bit

Handshake Xon Xoff       Modem HandShake OFF      GapTime 0

TERMINAL CONFIG:

NO local echo    No 8-bit ASCII    XmitFunc(A)=NO    SPOW(B)=NO

InhEOLWrp(C)=NO   Line/Page(D)=LINE    IndHndShk(G)=YES  IndD2(H)=YES

The Channel Simulator can be controlled from BASIC.  The following BASIC

program (HP BASIC 4.0 or newer, on Basic Workstation (running on a S200 or

S300 using the HP 98628A as mentioned above) illustrates by example how

this is done.

HP 11759C/D Remote Control Documentation

- 6 -

NOTE: Run the BASIC program before running RCHANSIM (11759C) or RGHOST (11759D)

on the PC.  The BASIC program needs to reset the serial interface before the PC

runs, or the PC will report a "Time out on serial write."  If that

happens, just press ESCAPE on the PC keyboard, and then run the BASIC

program first.  Then run the RCHANSIM program when the BASIC program

says to.

10  !                             RE-SAVE  "SAMPLE_PGM"

20  !

30  ! This program demonstrates how to download a setup and enter manual

40  ! commands remotely to the Channel Simulator software from BASIC on a

50  ! HP 9836A using a HP 98628A opt 002 datacom card with a HP 24542A cable

60  ! to the Vectra.  Communication happens at 300 baud so the Channel

70  ! Simulator software (on the PC) can keep up.

80  !

90  ! TO RUN THIS PROGRAM:

100 !   o First, connect the serial cable per the REMOTE.DOC file that

110 !     comes with the first HP 11759C/D INSTALL disk.

120 !   o Second, Load and RUN this program.

130 !   o When the program says, start the RCHANSIM or RGHOST on the PC.

140 !   o Then press CONTINUE on the BASIC computer.

150 !

160 ! The program runs in two phases.

170 !

180 !   (1) The first phase downloads a Simulation Test setup from the

190 !       DATA statements at line 'Sim_setup' to the Channel Simulator.

200 !

210 !   (2) The second phase prompts the user to enter some remote commands

220 !       from the keyboard.  This program then downloads it to the remote

230 !       controlled PC, and handles any resulting status or error messages.

240 !

250 ! THE IMPORTANT PART of this routine is the last subprogram in this

260 ! listing named 'Send_a_cmd()'.  It sends one command to the PC

270 ! and handles any expected return status and error messages by

280 ! printing them to the ATE controller's screen.

290 !

300 ! This was written to run on a HP 9836A, a series 200 computer, running

310 ! HP BASIC 4.0.  It should run on just about any version of HP "RMB" Basic

320 ! that can do the serial I/O.

330 !

340 DIM Cmd$[80]           ! Cmd to send to the Channel Simulator is put here.

350 !

360 GOSUB Main_init        ! Init SelectCode for Datacom port, init datacom card

370 !

380 DISP "Get RCHANSIM/RGHOST running on PC in main menu, then CONTINUE on this keybd:"

390 PAUSE

400 DISP

410 !

420 RESTORE Sim_setup      ! Set the READ statment to read a Rayleigh test setup

430 GOSUB Send_setup       ! Read each DATA statement, send the cmd to the PC.

440 !

450 GOSUB Indiv_cmd        ! Send indivual commands from the keyboard to the PC

460 DISP "Demo completed."

470 STOP                   ! Stop here, the demo has completed.

480 !

490 !

500 !

HP 11759C/D Remote Control Documentation

- 7 -

510 !!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

520 !!!!!!!          SUBROUTINES AND SUBPROGRAM FOLLOW    !!!!!!!!!!!!!!!!!!!!

530 !!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

540 !

550 Main_init:               ! !!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

560 ! Initialize the Data Com select code variable 'Sc', and also set up the

570 ! HP 98628A datacom card so it works using the correct datacom parameters.

580 Sc=20                    ! The select code of the HP 98628A interface card.

590 !

600 CONTROL Sc,0;1           ! Reset the card, flush any prev buffered traffic.

610 CONTROL Sc,8;2           ! Set the DTR line, this is very important to do!

620 !

630 ! The following 6 'CONTROL' statements set up the datacom parameters.

640 ! These are already set using the switches on the HP 98628A card, but

650 ! these commands are included here again for reference.

660 CONTROL Sc,20;7          ! 300  Baud, TRANSMIT

670 CONTROL Sc,21;7          ! 300  Baud, RECEIVE

680 CONTROL Sc,22;0          ! Protocall handshake disabled.

690 CONTROL Sc,34;2          ! 7 bits per character.

700 CONTROL Sc,35;0          ! one stop bit

710 CONTROL Sc,36;0          ! No parity.

720 RETURN                   ! End of 'main_init' subroutine, back to 'main'.

730    !

740    !

HP 11759C/D Remote Control Documentation

- 8 -

750 Sim_setup:      !        ! !!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

760     ! These data statements define word remote commands which are sent

770     ! from the HP 9836A (or equivalent ATE controller) to a PC which is

780     ! running the HP 11759C/D Channel Simulator software.

790  DATA "IDENTIFY?"          ! Query the Channel Simulator Software's rev

800  DATA "MENU?"              ! Query currently disp menu; should be MAIN

810  DATA "MENU SIMULATION"    ! Choose to run the Channel Simulation Test.

820  DATA "PATH 1"             ! Choose path one as the active path to work on,

830  DATA   "DELAY 5"          !   Set path 1 delay to 5 microseconds.

840  DATA   "DOPPLER 10"       !   Set path 1 Doppler to 10.0 Hz.

850  DATA   "ATTEN 2"          !   Set path 1 attenuation to 2 dB.

860  DATA   "SPECTRUM DOPPLER" !   Set path 1 to Doppler spectrum mode.

870  DATA "PATH 2"             ! Choose path two as the active path to work on,

880  DATA   "DELAY 10"         !   Set path 2 delay to 10 microseconds.

890  DATA   "DOPPLER 20"       !   Set path 2 Doppler to 20.0 Hz.

900  DATA   "ATTEN 4"          !   Set path 2 attenuation to 4 dB.

910  DATA   "SPECTRUM DOPPLER" !   Set path 2 to Doppler spectrum mode.

920  DATA "PATH 3"             ! Choose path three as the active path now.

930  DATA   "DELAY 15"         !   Set path 3's delay to 15 microseconds.

940  DATA   "DOPPLER 41.7"     !   Set path 3 Doppler to 41.7 Hz.

950  DATA   "ATTEN 6"          !   and set path 3's attenuation to 6 dB.

960  DATA   "SPECTRUM RAYLEIGH"!   Set path 3 to be spectrum type RAYLEIGH.

970  DATA "PATH 4"             ! Next, work on channel 2-- path 4= active path

980  DATA   "DELAY 20"         !   Specify 20 microseconds delay for path 4.

990  DATA   "DOPPLER 40"       !   Set path 4 Doppler to 40 Hz.

1000 DATA   "ATTEN 8"          !   Specify 8 dB attenuation for path 4.

1010 DATA   "CORRELATION .4"   !   Path 1 to path 4 Rayleigh correlation of 0.4

1020 DATA   "SPECTRUM DOPPLER" !   Set path 4 to Doppler spectrum mode.

1030 DATA "PATH 5"             ! Change the active (current) path to path 5 now.

1040 DATA   "DELAY 25"         !   and set it's delay to 25 microseconds.

1050 DATA   "DOPPLER 50.0"     !   Set path 5 Doppler to 50 Hz.

1060 DATA   "ATTEN 10"         !   and path 5's attenuation to 10 dB.

1070 DATA   "CORRELATION .5"   !   Path 2 to path 5 Rayleigh correlation of 0.5

1080 DATA   "SPECTRUM DOPPLER" !   Set path 5 to static Doppler mode.

1090 DATA "PATH 6"             ! Finally, set path 6 as the active path.

1100 DATA   "DELAY 30"         !   and set path 6's delay to 30 microseconds.

1110 DATA   "DOPPLER 41.7"     !   Set path 6 Doppler to 41.7 Hz.

1120 DATA   "ATTEN 12"         !   and path 6's attenuation to 12 dB.

1130 DATA   "CORRELATION .6"   !   Path 3 to path 6 Rayleigh correlation of 0.6

1140 DATA   "SPECTRUM RAYLEIGH"!   Set path 6 to be spectrum type RAYLEIGH.

1150 DATA ""                   ! This blank string tells output loop to stop

1160   !

1170   !

1180 Send_setup:               ! !!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

1190  ! Read DATA statements (set by previous RESTORE cmd) and send each string

1200  ! out to the PC that's running the Channel Simulator software.

1210  REPEAT                            ! Read each command that's part of the

1220    READ Next_cmd$                  ! setup to be downloaded from DATA.

1230    IF Next_cmd$<>"" THEN           ! If this cmd is not the end-of-list ""

1240      CALL Send_a_cmd(Sc,Next_cmd$) ! Then go ahead & send it to the PC.

1250    END IF                          !    & handle status, error returns.

1260  UNTIL Next_cmd$=""                ! Keep sending DATA cmds till @end

1270  RETURN

1280  !

1290  !

HP 11759C/D Remote Control Documentation

- 9 -

1300 Indiv_cmd:                 ! !!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

1310  ! Keep accepting one command from the keyboard, and sending it to the

1320  ! Channel simulator, until the user presses RETURN to stop.

1330  REPEAT

1340    LINPUT "Enter the remote command for the Channel Simulator: ",Cmd$

1350    IF Cmd$<>"" THEN                ! If user did not just press RETURN

1360      CALL Send_a_cmd(Sc,Cmd$)      ! then user entered a cmd; send it to PC

1370    END IF                          !   and handle status, error returns

1380  UNTIL Cmd$=""                     ! Keep asking user for more cmds

1390  RETURN                            ! until no more entered, then return

1400  !

1410  !

1420  END                               ! End of main & subroutines, begin

1430                                    ! defining the subprograms (only 1).

1440  !

1450  !

1460  !!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

1470  !!!!!!!                 SUBPROGRAM   DEFINED          !!!!!!!!!!!!!!!!!!!!

1480  !!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

1490  !

1500 Send_a_cmd:SUB Send_a_cmd(Sc,Cmd$) ! !!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

1510 ! Send a command to the remote controlled PC.  The 'Sc' param is the select

1520 ! code of the HP 98628A interface card that is installed in the HP 9836A

1530 ! (or equivalent) computer.  The 'Cmd$' is the command to sent to the PC.

1540 !

1550 ! ALGORITHM:

1560 !  Clear the input buffer--

1570 !    o First, the input buffer on the HP 98628A is cleared of any previous

1580 !      text that may have been sent by the Channel Simulator sw.

1590 !  Preprocess the command--

1600 !    o The command is trimmed of leading & trailing spaces.

1610 !    o If there was no entered command, then exit without doing anything

1620 !      else.

1630 !    o Print to this ATE controller computer's screen the cmd to be sent

1640 !      out.  If that command is a Ctl-[ (escape code), then just print

1650 !      <esc>.

1660 !  Send the Command to the PC--

1670 !    o If the command is a single character, then it is assumed to be a

1680 !      single-code command, so it is sent without a trailing <cr>.

1690 !    o Else the cmd is more than one character, so it is sent as a word

1700 !      command with a trailing <cr>.

1710 !  Get back the status & return messages (if any)--

1720 !    o If the sent command was not an ESCAPE, and it was only 1 character

1730 !      long, then the PC is not expected to return any sort of status, so

1740 !      subexit.

1750 !    o Else a possible error message, and ?? are expected, or just a ok is

1760 !      expected back from the PC.  So go into a loop & keep reading &

1770 !      printing any messages, until either the ?? error status or the OK

1780 !      status is read.  The expected return traffic is one of:

1790 !           OK<cr><lf>         or        <ErrMsg><cr><lf>??<cr><lf>

1800 !  Then exit back to the caller.

1810 !

HP 11759C/D Remote Control Documentation

- 10 -

1820 DIM A$[80]                  ! Input lines are read back into this string.

1830 !

1840 ! *** Clear the input buffer

1850 REPEAT                      ! Keep reading input buffer till it is empty.

1860   STATUS Sc,5;Inbuf_status   ! Check the status of the input queue.

1870   IF Inbuf_status>0 THEN    ! If the input buffer has some old data,

1880     ENTER Sc USING "K";A$   ! then do a read to clear out the old message.

1890   END IF

1900 UNTIL Inbuf_status=0        ! Repeat, in case there are more messages in

1910 !                           !   the buffer

1920 ! *** Preprocess the command.

1930 Cmd$=TRIM$(Cmd$)            ! Remove any leading & trailing spaces from cmd

1940 IF LEN(Cmd$)=0 THEN SUBEXIT   ! If the cmd being sent is nonexistent, exit.

1950 !

1960 IF Cmd$=CHR$(27) THEN       ! If the command being sent is the escape code,

1970   PRINT "<esc>"             ! print that the ESCAPE key (ctl-[) is cmd.

1980 ELSE                        ! ELSE assume the cmd is printable directly, &

1990   PRINT Cmd$                ! print the cmd which will be sent to the PC.

2000 END IF

2010 !

2020 !

2030 ! *** Send the Command to the PC

2040 IF LEN(Cmd$)=1 THEN              ! If cmd is a one-key command like ESCAPE,

2050   OUTPUT Sc USING "#,K";Cmd$     ! then output the cmd w/out trailing <cr>.

2060 ELSE                             ! Else assume it is a word command,

2070   OUTPUT Sc USING "+,K";Cmd$     ! and output it with a trailing <cr> char.

2080 END IF

2090 !

2100 !

2110 ! *** Get back the status & return messages (if any).

2120 IF Cmd$<>CHR$(27) AND LEN(Cmd$)=1 THEN  ! ..then it is a one-code command

2130   SUBEXIT                               ! which has no return status, so

2140 END IF                                  ! subexit w/out waiting for ok.

2150 !

2160 ! If got to here, then the command was either an ESCAPE key, or it was a

2170 ! command of more than one character in length (a word cmd).  So

2180 ! it should have some echo comming back from the PC-- and err & ??, or

2190 ! a query ret value (if a query) & the word "OK".

2200 REPEAT                            ! Repeatedly grab lines of serial port

2210   ENTER Sc USING "K";A$           ! Get next msg'from PC, cr/lf terminated

2220   PRINT A$                        ! Print the message (error or status).

2230 UNTIL A$="OK" OR A$="??"          ! Keep reading until error or ok status.

2240 SUBEND                            ! Finally, exit back to calling routine

HP 11759C/D Remote Control Documentation

- 11 -

:::::::::::::::::::::::::::::::::::::::::::::::::::::

:: INSTALLATION -- SOFTWARE PC's Channel Simulator ::

:::::::::::::::::::::::::::::::::::::::::::::::::::::

This section explains how to configure the Channel Simulator software on

the PC so it can be remotely controlled as mentioned above.

The Channel Simulator is initially called by changing to the Channel Simulator

program directory, and then typing RCHANSIM or RGHOST.  This calls a DOS batch

file, which starts the process.  The baud rate and other parameters are set in

this file, which is already set for using serial port COM1, with these datacom

parameters:

300 baud

No parity,

7 databits,

1 stop bit.

The RCHANSIM.BAT or RGHOST.BAT file can be edited with your favorite text

editor to change these settings.  It is important to keep the baud rate low

enough so multiple words can be interpreted without dropping characters.

Therefore, it is best to keep the baud rate at less than or equal to 300 baud

or the remote commands may not be recognized.

To test out the link, do the following:

1. cd to the directory where you installed the Channel Simulator

programs (the "program directory").  If you have not installed

the software yet, please see the README.NOW file on your

Channel Simulator's Install disk for installation directions.

2. Run the BASIC program, or start the terminal emulator, so the

DTR line is pulled high.  This needs to be done before the

Channel Simulator is started in remote control mode.

3. type RCHANSIM

-or-

type RGHOST

4. Send "IDENTIFY?" to the PC from the ATE controller, with a

terminating CR (no linefeed).  You should see the IDENTIFY?

echoed at the bottom of the screen, and the revision of the

Channel Simulator software will appear.

IN CASE OF DIFFICULTY:

o If the computer keeps displaying 'timeout' messages, then the ATE

controller has not raised the Vectra PC's DSR line.  Raise the DC

voltage of this line, and the timeout messages will stop.  Or, enter a

Ctl-C (before any other key presses) from the PC keyboard to stop the

timeout messages and exit the program.

o If the baud rate is wrong, etc, the above characters will not be echoed.

In that case, you can always press Q on the PC keyboard to stop the

program.

o Sometimes it is useful to connect an external terminal's receive line

to the transmit line of the ATE controller to make sure it's sending

in the right data format, baud rate, etc.  Likewise, the terminal transmit

line could be connected to the PC's serial line to drive the

Channel Simulator directly.

o Make sure the wiring mentioned above, or it's equivalent, has been

followed.  Faulting wiring will never work, no matter what the datacom

parameters are.

HP 11759C/D Remote Control Documentation

- 12 -

:::::::::::::::::::::::::::::::::::::

:: REMOTE COMMANDS Introduction -- ::

:::::::::::::::::::::::::::::::::::::

This section summarizes the commands that are used by a remote controller

to serially remote control the HP 11759C/D software on a PC.

WORD COMMAND PROGRAMMING

The primary programming method involves sending word-commands to the PC Channel

Simulator software.  Parameters are changed by specifying a new current path,

and then the parameters to change in that path, by name.

After each word command is sent, at least one message is always returned

to the ATE controller.  This message traffic is used for error checking, and

indicate to the ATE control program when the next command can be sent.  See

"Remote Command Error Handling" below for more details.

VERIFICATION OF COMMANDS

Any remote word command can be typed into the PC keyboard while the

software is in local control (ie, started with CHANSIM or GHOST, not with

RCHANSIM or RGHOST.)

The command is echoed at the bottom of the screen, just as it would be

if the software were in remote control mode.  This can be used to check the

syntax of a command that is not working over the serial remote control.

ONE-KEY CONTROL PROGRAMMING

This auxilary remote control control method begins with one-key control codes

which move the cursor to choose a parameter for modification.  Numbers are

then sent to change the selected parameter, similar to the local front panel

operation.

The Channel Simulator software does not echo back any characters in response

to a ONE-KEY command.  The only indication a one-key command has been

accepted is the action happening on the PC display.  Those programs which

require status & error reporting from the Channel Simlulator should use the

Word Commands mentioned in the previous section.  Use of the word commands is

the prefered remote control method for the Channel Simulator software.

One-key control codes are listed at the end of this document.

::::::::::::::::::::::::::::::::::

:: REMOTE COMMANDS Syntax Rules ::

::::::::::::::::::::::::::::::::::

NOTATION:

In the following, notation is used to indicate certain control characters

from the ascii table.  For example, a decimal value of 27 corresponds to

the ESCAPE key in ASCII.  Other values, and the notation to indicate them in

this document, are:

decimal 13 = Carriage Return.. <cr>    decimal 10 = NewLine ....... <lf>

decimal 10 = LineFeed ........ <lf>    decimal 27 = Escape Key..... <escape>

HP 11759C/D Remote Control Documentation

- 13 -

Another notational comment-- quote marks are used to demark what would be sent

to or read back from the Channel Simulator over remote control.  Those quote

marks should never actually be sent to, or perceived as being sent by, the

Channel Simulator (except for the profile titles).  For example, sending a

"IDENTIFY?<cr>" to the channel simulator means sending an I, a D, an E, etc,

up to the <cr>.  The only exception to this is the handling of titles for

profiles.  These titles are enclosed in quote marks when received and sent.

See the tables below for details.

SYNTAX RULES:

Several rules must be followed when sending remote commands to the

Channel Simulator software, in addition to the wiring and datacom

parameters described above.

In General;

o The command(s) can be sent in upper case or lower case; they will

automatically be changed to upper case by the Channel Simulator.

o The PC keyboard will not read keys during remote control of the channel

simulator, except for Q and <esc> and Ctl-C (see next note).

o If the remote control datacom connection fails for some reason, the

'Q' and the <esc> keys will still work on the PC keyboard to escape

out of any test, and then Quit the main program.  This avoids having

to reboot the PC in the event of difficulty.

For word commands (they are listed later in a table);

o Each word command must be followed by a <cr>, or it will not be

recognized by the Channel Simulator.

o Word commands are always echoed on the bottom of the PC display as they are

received from the remote controller.  Returned messages from the Vectra PC

to the remote controller are also shown on the Vectra PC display bottom

line.  The remote message traffic may be monitored by watching this display

line.

o Each word command should check the return status after each

command to catch errors and avoid overwriting the Channel Simulator

buffer.  See "Word Command Error Handling" below.

o Each returned query message from the PC back to the ATE controller is

always terminated by a <cr><lf> sequence.

o At lease one space must separate the command from it's argument, if an

argument is given.  Example/ DELAY 55<cr> has a space between the word

"DELAY" and the number "55".

o Any suffix after a numeric argument is ignored.  For example, the command

"DELAY 55 ms<cr>", the 55 would be interpreted in _microseconds_, and

the ms would be ignored (no error is generated).

o The numeric argument can be in either fixed or floating point notation.

For example, "DELAY 5.5E+1<cr>" would be interpreted as the same delay

as the command "DELAY 55<cr>".

HP 11759C/D Remote Control Documentation

- 14 -

Word Command Rules (cont):

o The units of the arguments are limited to Hertz for Doppler, and

microseconds for delay.  If kilometers per hour and kilometers must

be used for Doppler and delay, use the cursor motion single-key

commands to move the cursor to the correct cell on the test's chart,

and send the number to make the change.  (See the timing caution for

single-key commands below).

o For queries (like "MENU?") there should be no space between the command

MENU and the question mark.

:::::::::::::::::::::::::::::::::::

:: REMOTE COMMAND ERROR HANDLING ::

:::::::::::::::::::::::::::::::::::

Any errors resulting from the multi-command words are printed back to

the ATE controller by the Channel Simulator software on the PC as soon

as they are detected.  The error is immediately followed by "??<cr><lf>".

This return message(s) should be checked (by the ATE remote control program)

after each command to make sure no errors occurred.

To summarize, error checking after each command is accomplished by

waiting after the sending of each command for either

<Error or Warning Msg><cr><lf>

[ <Error or Warning Msg><cr><lf>  ]

[ <Error or Warning Msg><cr><lf> ]

and  ??<cr><lf>

or

OK<cr><lf>

or

<Query Msg><cr><lf> and  OK<cr><lf>

from the Channel Simulator before sending the next command.  The types

of errors, and how to correct them, are listed in last section of this

document.  The rectangular braces '[' and ']' in the above list indicate

that between one and ten error messages may appear before the ?? status

is finally sent.  All errors beyond the tenth encountered error are not

reported over the remote control serial link.

:::::::::::::::::::::::::::::::::::::::::

:: REMOTE COMMANDS SUMMARY OF COMMANDS ::

:::::::::::::::::::::::::::::::::::::::::

MAIN MENU COMMANDS:

--------------------      -------    -------------------    -----

FUNCTION              LOCAL     WORD REMOTE COMMAND    NOTES

--------------------      -------    -------------------    -----

Query the software version              IDENTIFY?            2

Query the Current menu                  MENU?                3

Call up the Simulation Menu  S          MENU SIMULATION

Call up the Travel Menu      T          MENU TRAVEL          1

Quit the Channel Simulator   Q          EXIT

HP 11759C/D Remote Control Documentation

- 15 -

Main Menu Remote Command Notes:

1. When the Travel test is run, the last saved setup is recalled and

run.  Any new setup must be entered and locally run (from the PC's

keyboard) to completion from local PC keyboard control before it can

be used remotely.  Only one such setup can be saved using the travel

test.

2. The SW version query returns

$ Revision: N.M $<cr><lf>

where N and M are digits indicating the version of the Channel

Simulator software on the PC.  This corresponds to the number in

the upper left corner on the various menus.

3. The Query of the current menu returns "MAIN<cr><lf>" for the main menu.

SIMULATION TEST MENU COMMANDS:

--------------------      -------    -------------------          -----------

FUNCTION              LOCAL     WORD REMOTE COMMAND             NOTES

--------------------      -------    -------------------          -----------

Query the SW version      See Display     IDENTIFY?                1

Query the Current menu    See Display     MENU?                    6

Return to main menu       <ESCAPE> 	   EXIT

Reset the chart params Ctl-<BackSpace>    RESET

Set a New current path  (CursorMotion)    PATH <n>

Query the current path    See Display     PATH?                    10

Query Standard  profile title    [Alt-P]  SPROFILE? <number>       16

Query User Defined profile title [Alt-P]  UPROFILE? <number>       8,16

Save a User Defined profile      [Alt-P]  USAVE <number> "<Title>" 7, 16

Recall a User Defined profile    [Alt-P]  URECALL <number>         2,3,7,16

Recall a Standard Profile        [Alt-P]  SRECALL <number> <HzRayleigh>  2,3,

and 7,16

FOR THE CURRENT PATH - - - - - - - - - - - - - - - - - -           9

Query channel RF freq    See Display    FREQUENCY?                14, 15

Set Spectrum type      (CursorMotion)   SPECTRUM {OFF DOPPLER RAYLEIGH} 2,3,4

Query Spectrum type      See Display    SPECTRUM?

Set delay              (Cursor Motion)  DELAY <usec>              11

Query delay 		    See Display   DELAY?                    11,15

Set Doppler 		  (Cursor Motion) DOPPLER <Hz>              2,3,13

Query Doppler	            See Display   DOPPLER?                  13,15

Set atten 		  (Cursor Motion) ATTEN <dB>                12

Query atten 		    See Display   ATTEN?                    12,15

Set Rayleigh correlation (Cursor Motion)CORRELATION {.1 .2 .3 .4 ... .9 1.0 }

Note Num--->   4, 5

Query Rayleigh correlation  See Display CORRELATION?              15

HP 11759C/D Remote Control Documentation

- 16 -

Notes for Simulation Test commands:

1. The SW version query returns $ Revision: N.M $<cr><lf>, where N and M are

digits indicating the version of the Channel Simulator software on the PC.

This corresponds to the number in the upper left corner of the display.

2. Warning:  If the Configuration menu has the Data directory specified

differently from the Program directory, then all files ending in .IQ will

be *DELETED* from the Data directory whenever a new non-zero Doppler is

specified for a Rayleigh path, (or when a path with a non-zero Doppler

value is made Rayleigh).  This is also true for local operation from the

Vectra PC keyboard.

3. Certain Rayleigh integrity checks are made whenever a path is made

Rayleigh, or when the Doppler is changed on a Rayleigh path.  These

checks are:

-- All Rayleigh paths must be running the same Hz Doppler.  The

software manages these changes automatically and issues a warning

that a parameter was changed.

-- The Rayleigh path Dopplers can not have a fractional part when

converted to the equivalent kmh at a 900 MHz RF frequency.  The

software rounds the Doppler automatically and issues a warning

that a parameter was changed.

-- The Rayleigh fading .IQ datafile must already be built for

the Doppler on any Rayleigh path (except 0 which does nothing).

-- The Rayleigh .IQ file must appear on the Data directory, or there

must be enough space on the drive to accept the copy of the .IQ file

from the Program directory after automatic removal of ay previous .IQ

files.  The software will automatically copy the .IQ file to the data

directory, and issue a status message that it happened.

4. The Rayleigh correlation will only be applied under the following:

+ Between path 1 and 4,  -or-  between path 2 and 5,  -or- path 3 and 6.

It is not possible to correlate any other path combinations, only

these three.  All 3 pairs can be pair correlated at the same time,

but each pair combination will be uncorrelated with the other pairs.

+ The correlation will only happen between the two paths of the pair if

BOTH the paths are in the Rayleigh spectrum mode.

+ The correlation number on the chart is marked with parentheses if either

of the paths are NOT in Rayleigh mode, which indicates that the

correlation is not happening between those two paths.

5. The CORRELATION numeric must be one of the following:

0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0.

6. The Query of the current menu returns "SIMULATION<cr><lf>" for this menu.

7. The entire chart setting (except for the Configuration menu specified

RF frequencies) may be recalled from disk using the URECALL <n>

command for user defined profiles, or SRECALL <n> <HzRayleigh> for

the predefined Standard Profiles, where 'n' is one of 10 possible

disk file "folders" of settings.  If a Rayleigh Hz Doppler value is

not specified when recalling one of the standard profiles, an error

message ("need floating arg") is generated.

8. If the existing chart settings are saved using USAVE <n> "<text>",

the 'text' is a quote enclosed description of up to 19 characters

that are to be identified with that setting, and may be queried with

UPROFILE?  <num>.  More characters can be used in the title if you

use the alt-P profile menu.  All title characters entered remotely

will be automatically converted to uppercase.

9. Many of the commands only effect the settings for the *current* path.

Therefore, the current path must be specified with the PATH command

before using these commands to change a specific path.

10. The current path may be queried using the PATH? query, which returns

"PATH <N><cr><lf>", where N is an ascii integer between 1 and 6.

11. The <usec> argument for the DELAY command must be in

microseconds.  It is not possible to enter a delay in kilometers

using word commands.

HP 11759C/D Remote Control Documentation

- 17 -

CHANNEL SIMULATION MENU NOTES (CONT):

12. The ATTEN command's <dB> argument must be in dB.

13. The DOPPLER command must be in Hz.  It is not possible to enter kmh

Dopplers using the word commands.

14. The preference menu must be used to specify the RF frequency of the

channel.  This must be done under local control from the PC's keyboard.

15. All parameter queries are in the same units that are used to set the

command remotely.  The units of the FREQUENCY query (which can not be set

remotely) is Hz.

16. Profiles work similarly to operation from the PC keyboard.  The <number>

is an integer between 1 and 10 inclusive.  A Title query UPROFILE? <num>

or SPROFILE? <num> will return the user or standard profile title

respectively, as a string included in quote marks.  Any warnings

found in the recalled state are printed with an OK, and any errors

are printed with ??.

TRAVEL TEST COMMANDS:

--------------------  -------  -------   -------------------------

FUNCTION          LOCAL   ONE-KEY            WORD

--------------------  -------  -------   -------------------------

Return to main menu   <ESCAPE> <ESCAPE>         <none>

Notes:

o The Travel Test always runs the last saved setup.  There is no way

to specify a new scenario, vehicle speed, etc., over remote control.

Full functionality is available by local control from the Vectra PC

keyboard.

o To change Travel Test parameters, re-run the Travel Test under local

control (saving setup) and let the test FINISH.  This allows each path

attenuation at each simulation time to be computed and saved into the

scenario file.  Then, restart the Channel Simulator remotely and call the

Travel Test menu.

o The remote control access to the Travel Test is minimal.  The only

available remote control command is to exit the test by sending the

one-key <esc> (escape) key, with no terminating <cr>.  The main menu will

return an OK to the remote controller after the Travel Test has been

exited with the <esc> (escape) key.

HP 11759C/D Remote Control Documentation

- 18 -

::::::::::::::::::::::::::::::::::::::

:: REMOTE COMMANDS-- Error Messages ::

::::::::::::::::::::::::::::::::::::::

The following is a list of all possible error messages, and what those

messages mean.  Hints are provided to suggest how to work around the

error.  As mentioned numerous times above, errors are only returned for

the word commands (listed above).

"ERROR: .IQ Copy to fast drive failed."

The Rayleigh data file (.IQ file) could not be copied to the fast drive.

See the errors "ERROR: Data Dir too small for IQ file.",

"ERROR: Fast Directory does not exist." and

"ERROR: Insufficient DOS memory" for possible explanations of what may

have gone wrong.

"ERROR: .IQ file header is bad or missing."

The selected Rayleigh file does not contain a valid format.  The file has

become corrupted and must be rebuilt using the IQMAKE utility.

"ERROR: .IQ file's data rate is out of rng."

The selected Rayleigh file contains an "out of range" output data rate.

This file cannot be used by the existing 11759 hardware.

Try rebuilding the Rayleigh file using the IQMAKE utility.

"ERROR: 11759 error, BUSY line stuck high."

When the HP 11759 was being reset by the computer, it's handshake line

was stuck in a high state.  This is a very rare error.

This is indicative of a hardware problem.  There is a FIFO (first-in,

first-out) memory whose data could not be flushed.  That is, after an

attempt to clear the data out of the FIFO, the FIFO still reported that

it had more than half of it's memory filled with data.

To correct his problem, try doing a Control-[BACKSPACE] to reset the

test and then rerunning the same sequences that caused the problem the

first time.  Power cycling the HP 11759 may also clear the problem

up temporarily.  Also, some connections may have become unseated at the

rear of the instrument, the back of the HP 11759, or inside the HP 11759

itself.  Please follow the Service manual procedures if the HP 11759

needs to be opened to reset the connections.

HP 11759C/D Remote Control Documentation

- 19 -

"ERROR: 11759 error, BUSY line stuck low."

When the HP 11759 was being reset by the computer, it's handshake line

was stuck in a low state.  This is a very rare error.

This is indicative of a hardware problem.  There is a FIFO (first-in,

first-out) memory which could not be filled more than half full of data

when it was not being read, and it was was being written to by the

computer.  It continued to report an "empty" condition well after

it should have been completely filled up.

To correct his problem, try doing a Control-[BACKSPACE] to reset the

test and then rerunning the same sequences that caused the problem the

first time.  Power cycling the HP 11759 may also clear the problem

up temporarily.  Also, some connections may have become unseated at the

rear of the instrument, the back of the HP 11759, or inside the HP 11759

itself.  Please follow the Service manual procedures if the HP 11759

needs to be opened to reset the connections.

"ERROR: 11759 is either off or unconnected."

This indicates that the HP 11759 is either powered down, or it is not

connected to the PC.  The handshake controller in the HP 11759 is not

configuring itself correctly during a HP 11759 reset sequence.

Correct this by verifying that the HP 11759 is powered up, that is

connected to the computer via the parallel printer port cable, and

that the cable is firmly connected at both ends.  Also make sure that

this parallel cable is connected to the correct LPT printer port on

the computer.  The LPT port which is used on the computer must match

the port specification in the HP 11759 software's Configuration menu.

Consult your computer manual to verify configuration of the parallel

printer port, or try running the PORTTEST program on the computer.

PORTTEST is documented in the service manual.

"ERROR: 11759 is not connected to computer."

This indicates that the computer was unable to find any HP 11759

connected to the LPT port which was specified in the HP 11759 software

Configuration menu.  The parallel printer port Select line was found

to be high, and it is normally grounded through the HP 11759 when

it is connected.  This is pin 25 on the parallel printer port cable.

Correct this by verifying that the HP 11759 is powered up, that is

connected to the computer via the parallel printer port cable, and

that the cable is firmly connected at both ends.  Also make sure that

this parallel cable is connected to the correct LPT printer port on

the computer.  The LPT port which is used on the computer must match

the port specification in the HP 11759 software's Configuration menu.

Consult your computer manual to verify configuration of the parallel

printer port, or try running the PORTTEST program on the computer.

PORTTEST is documented in the service manual.

HP 11759C/D Remote Control Documentation

- 20 -

"ERROR: 11759 RearPanel 10MHz is wrong freq."

The HP 11759 rear-panel reference frequency is either at the wrong

frequency, or it's levels are incorrect.

The computer determines this by loading the HP 11759 internal FIFO

with data, and then clocking it out at a time which is dependent on

the rear panel 10 MHz frequency.  If the fifo empties too quickly or

too slowly relative to the computer's internal timer, then the

computer concludes that the frequency is incorrect.

This can usually be corrected by insuring a 10 MHz signal is appearing

at the rear panel 10 MHz setup.  Please consult the Installation

part of the manual for details on the frequency accuracy and level

requirements of the rear panel TIMEBASE input signal to the HP 11759.

"ERROR: 6x6 Correlation tried with 6 paths."

The correlation mode of 6X6 was attempted with a 11759 that only has

6 paths.  6X6 correlation mode can only be used on a 12 path system.

If you have a 12 path system, use the Configuration menu to set

the number of units in the system to 2.  If you have only 6 paths, only

use correlation modes of 3X3 or NONE.

"ERROR: Argument has too many digits."

The correlation factor argument for the correlation command was

specified with more than one digit after the decimal point.  The

only allowed correlations are 0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7,

0.8, 0.9, 1.0.  Any number less than zero or greater than one would

be "out of range".

To correct this, make sure the numeric argument for the correlation

command is from the above list without any extra digits.

"ERROR: Argument is out of range."

The numeric parameter that was supplied with the command was either

too big, or too small.  For example, a SMOD1:CORR -10 command would

cause this error, because the only allowed correlations are 0, 0.1,

0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0.  Any correlation factor

less than zero or greater than one would be "out of range".

To correct this error, make sure the numeric argument of the command

being sent is within the range indicated in the above documentation

for that menu's command.  The range of any parameter may be easily

checked by running CHANSIM (11759C) or GHOST (11759D), locally, on the

Vectra PC, and entering the out of range parameter manually.

A pop-up window will appear with a detailed description of the error.

HP 11759C/D Remote Control Documentation

- 21 -

"ERROR: Computer too slow for realtime fade"

The computer was not sending the Rayleigh or IQ fading values (or

the dynamic test settings) to the HP 11759 to maintain real time

fading.  The computer was not able to send data quickly enough

at some point in the simulation.

The result of this would be pauses in the fading scenario, where

the hw settings would remain at some value for longer than they

should.  Since it could cause in out-of-spec fading statistics

and bandwidth problems, the computer flags it as an error.

This is often caused by running with a computer which is too slow.

It can also be caused by the presence of other "TSR" (Terminate and

Stay Resident) software in the computer which is using the cpu long

enough to interupt the average fading data rates to the HP 11759.

LAN software, hard disk interuptions, etc can all cause this.

Corrective actions include using a faster computer, making sure the

fading data is read from a RAM disk when possible, and making sure

that no other programs are running simultaneously when the HP 11759

is attempting realtime fading.

"ERROR: Data Dir too small for IQ file."

The Rayleigh data file could not be copied from the Program directory to

the Data directory because there was not enough space on the Drive which

holds the Data directory, after deleting all Data directory .IQ files

which are deletable.  This copy operation was needed to get the Rayleigh

fading datafile to the fast Data directory for one or more of the paths

which is in the Rayleigh fading mode.

To correct this, remove files or redeclare the RAM disk such that,

after all files ending in .IQ on the Data directory (that do not have

their DOS Read Only attribute set) for have been removed, there is

at least 3.2 Mega-bytes of available space to accept a new .IQ file

via a file copy from the Program directory.  In addition, the DOS

command ATTRIB can be used to make sure the previous Data Directory

.IQ files are not read only.  This allows the software to delete the old

.IQ files to make room for the new .IQ file copy.

NOTE:  The program automatically removes any file ending in .IQ from

the data directory IF the data directory is something other than the

program directory.  This is done to make room for the copying of the

new .IQ file to the data directory.  This .IQ file deletion mechanism

will not work if the .IQ file attributes have been set to READ ONLY.

Also, an "access denied" message shows on the Vectra PC during the

copying if .IQ files are set read only.  That can be ignored if

there is enough Data Dir space for the copy operation to work.  If

there is not enough space, then the program will terminate with a

message saying that the datafile could not be found on the data directory.

If the Configuration menu has been set up with the same Data directory

and Program directory, then this .IQ file copy is not done, since

the .IQ file should already be on the program directory.  However,

the Data directory can not be made the same as the Program Directory

(in the Configuration menu) unless the Program directory has a fast

disk cache for the Rayleigh data.  See the Install pgm for more info.

HP 11759C/D Remote Control Documentation

- 22 -

"ERROR: Doppler specified while in Rayleigh"

The doppler frequency may not be changed on a path running rayleigh.

To change the rayleigh doppler frequency, change the active file name

using the remote SMOD[n]:RAYL:FILE command.

"ERROR: Fast Directory does not exist."

The requested Rayleigh fading data file couldn't be copied to the

fast data directory because the data directory does not exist.  The file

was being copied to the data directory so it could be accessed

rapidly for simulating the Rayleigh in real-time.

To correct this condition, go back to the Configuration menu and

follow the prompts to specify a new Data directory.  Recheck the

data directory existence from the DOS prompt if desired.

"ERROR: Illegal argument."

The argument for a command was incorrect or out of range.  For example,

this error is caused be all of the following mistakes:

o A new spectrum was specified that was not OFF, RAYLEIGH, DOPPLER or

PHASE.

o A new delay resolution was specified that was not LOW or HIGH.

Modify your remote command syntax to use the arguments allowed.  See

the remote description for each command for details.

"ERROR: Insufficient DOS memory."

The program couldn't run anymore because there wasn't enough memory

in the computer.  This refers to the lack of "core" memory, which is

commonly called conventional memory, that's in the 640K region on a PC.

To resolve this problem, make sure your PC has 640Kbytes of RAM.

If it does, then review the documentation for your computer and your

operating system for directions on increasing the amount of conventional

memory that is available for DOS applications.  This may mandate

changes to the configuration files CONFIG.SYS and/or AUTOEXEC.BAT.

HP 11759C/D Remote Control Documentation

- 23 -

"ERROR: Invalid string data."

The string argument for a command was either missing, incomplete,

or of a bad format.  The string argument must be enclosed in single or

double quote marks.  If the string, begins with one type of quote it

must end with the same type of quote.

To correct this condition, make sure the string is printed to the PC

with leading and trailing quote marks, like this example:

SMOD1:RAYL:FILE "RAY834.IQ"

"ERROR: IQ file not found on HardDrive dir."

An attempt was made to operate a Rayleigh mode path with a rayleigh

file which was not been built.  This may happen when a path is

turned to Rayleigh, or the Doppler of a path which is already in

Rayleigh mode is changed to use an unbuilt Rayleigh file.  The Rayleigh

test can only be started using already-built Rayleigh .IQ data files.

To correct this, either specify a speed that already exists or use the

IQMAKE utility to build the data file.  Use the DOS command DIR *.IQ in

the program directory will show what data files already exist.

Check that the program directory is correct in the Configuration menu.

If the error happened during a User Profile recall, then a user

defined profile call should be attempted locally.  A more detailed

error message will show the offending entry.  That Rayleigh file could be

built with IQMAKE, the .IQ file could be copied to the data

directory, or a text editor could be used to change the settings in

the user defined profiles file CHANSIM.PRO (11759C) or VIDEO.PRO (11759D)

in the program directory.

"ERROR: Missing 11759 rear panel 10MHz sig."

The HP 11759 rear-panel reference frequency is missing or it is at

too low of a power level.  It could also be the wrong frequency

(less than 1 MHz or more than 22 MHz).

The computer determines this by loading the HP 11759 internal FIFO

with data, and then clocking it out at a time which is dependent on

the rear panel 10 MHz frequency.  If the fifo never empties, then the

computer concludes that the timebase frequency is missing.

This can usually be corrected by insuring a 10 MHz signal is appearing

at the rear panel 10 MHz setup.  Please consult the Installation

part of the manual for details on the frequency accuracy and level

requirements of the rear panel TIMEBASE input signal to the HP 11759.

HP 11759C/D Remote Control Documentation

- 24 -

"ERROR: Needs a character argument."

A command was specified with either an incorrect argument, or with

a missing argument.  The command requires a legal unquoted character

string.  For example, LOW, HIGH, RAYLEIGH, etc. are character arguments.

To correct this, make sure a valid unquoted character string is supplied

after the command.

"ERROR: Needs a numeric argument."

A command was specified with either an incorrect argument, or with

a missing argument.  The command requires a legal numeric value.

For example, the number may be of the form 12, 12.4, -12.4,

or -1.24E+1, etc.

To correct this, make sure a valid numeric value is supplied after the

command.

"ERROR: Needs query or argument."

Examples of what can cause this error:

o A spectrum command was used to switch the spectrum mode of the

currently active path, but the name of the new spectrum mode was

not specified.

o A new correlation factor was attempted, but no correlation number

was included after the command.

o A query was desired but the ? was missing from the mnemonic.

Modify your remote command syntax to include the argument that is

specified for that command or to include the ? if a query is desired.

"ERROR: No argument allowed with query."

A query syntax contained an argument after the ?, but this

query does not allow an argument.

To correct this, modify your remote query syntax to remove the argument.

"ERROR: No argument is allowed."

A command syntax contained an argument, but this command does not allow

an argument.

To correct this, modify your remote syntax to remove the argument.

HP 11759C/D Remote Control Documentation

- 25 -

"ERROR: Profile file was not writable."

The User profile file CHANSIM.PRO (11759C) or VIDEO.PRO (11759D) was

not writable when the user profile command was attempted to save a

user defined profile.

To correct this error, use the various DOS commands to make sure

that the file is not read-only, and use the DOS CHKDSK command

to make sure that there is enough space on the program directory

for the file.

"ERROR: Profiles file missing or bad format"

While recalling either the user defined profile, or a standard profile,

there was a problem while reading the file.  The file was either not

available for reading, or the contents of the file contained a syntax

error.

To correct the problem, try recalling the profile while under local

control from the PC keyboard.  Use the Alt-P profile menu to recall

the same type of profile which caused the error-- standard or user

defined.  Then enter the number of the profile, and then select the

rayleigh data file if recalling a standard profile.  The error message

will provide more information on any syntax error, and then guide you

to one of the following corrective actions:

o Re-build a new all-blank user defined profiles

file CHANSIM.PRO (11759C) or VIDEO.PRO (11759D)

o Recopy the STANDARD.PRO (11759C) or S_GHOSTS.PRO (11759D) standard

profiles file from the install disk.

"ERROR: Query DEADLOCKED."

This error can occur only if a query's response is too long for the

11759 to handle.  This should never occur; but if it does

try entering the same command under local control from the PC keyboard.

The error window will provide more information on any syntax error,

and then guide you to a possible corrective action.

"ERROR: This is a Command only."

A command which can never be a query was received with

a question mark after it's name.

For example, *RST must always be sent without a trailing question

mark, because it can only be used as a command.

To correct this, make sure there is no ? following the command.

See the individual command and query descriptions for details.

HP 11759C/D Remote Control Documentation

- 26 -

"ERROR: This is a Query only."

A command which is only supposed to be a query was received without

a question mark after it's name.  Or, an argument may have been

supplied for the query, where only a query is allowed.

For example, *IDN must always be sent with a trailing question

mark, because it can only be used to query something.

To correct this, make sure you only send the query form of the command.

See the individual command and query descriptions for details.

"ERROR: Title argument has too many chars."

While saving a user defined profile, the title argument was too long.

The title can be a maximum of 55 characters.

The title argment must be enclosed in double quote marks.

To correct this condition, make sure the title is printed to the PC

with leading and trailing quote marks, like this example:

MMEM:STOR:STAT 3,"Saving a really long title into register 3"

"ERROR: Title argument is bad or missing."

While saving a user defined profile, the title argument was either

missing or incomplete, or of a bad format.

The title argment must be enclosed in double quote marks.  To

correct this condition, make sure the title is printed to the PC

with leading and trailing quote marks, like this example:

MMEM:STOR:STAT 5,"Saving to reg 5"

"ERROR: Too Many Characters"

The Vectra PC received too many characters on a single line (lines are

ended by a <cr> in serial control mode and by a <lf> in HP-IB control

mode).  The input buffer on the 11759 software is 80 characters.

For example, this error could occur when specifying the title for the

MMEM:STOR:STAT command, which titles the profile that is being saved.

The title is limited to 55 characters.  Staying withing this limit

will not overflow the 80 character input buffer, but if a longer title

is attempted the buffer could overflow.

Correct the problem by following the command syntax, and limiting

the title length to the allowed size.

HP 11759C/D Remote Control Documentation

- 26.1 -

"ERROR: Too many settings in same timeframe"

The computer tried to send too many hardware parameter settings to

the HP 11759 within the time period required for those settings to

be implemented.

A real time fading scenario which downloads data from a file requires

that all the hardware settings for one particular instant in the

simulation be downloaded to the computer in one group, which is called

a logical "frame".  If too many of these settings are included in a

frame, then the HP 11759 won't be able to interpret and process each

of the settings before the next frame arrives.

This can be corrected in the dynamic test by sending fewer hardware

settings in the same frame.  The update rate may also be slowed down,

which gives the HP 11759 more time to interpret each frame of settings.

The Rayleigh and/or IQ fading should not show this error, since those

settings are all predetermined at the factory.  In this case, the

HP 11759 may require service.

"ERROR: Unknown Command"

The command that was entered is not known to the currently active

Channel Simulator module.  Some commands work in the main menu,

and other commands work in the test menus.  The right commands

must be used in the right menus.

To correct this error condition, re-send a command that works

in the currently active screen.  Check for misspelled words.  Check for

a missing <lf> (for HP-IB) or <cr> (for serial) at the end of the program

line.  The type of screen that is currently active may be queried using

the PROG:NAME? word command.

It will return MAIN, TRAVEL, DYNAMIC, GHOST or SIMULATION.

The commands allowed for each screen are listed with the detailed command

descriptions.

"ERROR: <other error>"

The other types of errors are related to the datacom port, or the

parser in the Channel Simulator software.  They should be self

explanatory--

o Characters overflowing the buffer

o Datacom timing or framing error

HP 11759C/D Remote Control Documentation

- 26.2 -

"WARNING: Delay value was rounded."

The delay value for a path was rounded.  If using LOW delay resolution

mode, the delay was rounded to the nearest 50 ns.  If using HIGH delay

resolution, the delay was rounded to the nearest 1 ns.

This warning can be safely ignored, as long as the rounded value meets

the simulation requirements.

"WARNING: No correlation, pair not Rayleigh"

Rayleigh correlation was specified for a path whose Spectrum mode is

not Rayleigh, or whose associated path in the other channel is not

Rayleigh.

This warning may only be ignored if correlation is not expected between

the current path and it's pair, or if both paths in the pair will be

made Rayleigh before correlation simulation is desired.

When the correlation mode is 3X3,

the paths in each path pair of 1&4, 2&5, and 3&6 may only be pairwise

correlated if both members of the pair are in the Rayleigh spectrum

mode.  For example, if path 3 is in Rayleigh spectrum mode and path

6 is in Rayleigh spectrum mode, then the correlation for path 3 or

path 6 may be specified.  If either path is either OFF or DOPPLER,

then this warning message will be issued.

HP 11759C/D Remote Control Documentation

- 27 -

:::::::::::::::::::::::::::::::::::::

:: ONE-KEY REMOTE CONTROL COMMANDS ::

:::::::::::::::::::::::::::::::::::::

This section describes the one-key remote control codes for the Channel

Simulator.  As mentioned previously, the word commands are preferred for

remote control, since they allow command holdoff and error reporting.

The one-key commands for cursor motion & changing values on the Channel

Simulation chart are:

Decimal  1 = HOME key........... <Ctl-a>    <--- UpperLeft on chart

Decimal  2 = Cursor Back Key.... <Ctl-b>    <--- Move Left 1 field

Decimal  6 = Cursor forward key. <Ctl-f>    <--- Move right 1 field

Decimal 12 = Reset Chart key.... <Ctl-n>    <--- HW reset of test.

Decimal 14 = Cursor NextField... <Ctl-n>    <--- Move down 1 field

Decimal 16 = Cursor PrevField... <Ctl-p>    <--- Move up 1 field

Decimal 96 = BackQuote key (`)..   `        <--- Path OFF or DOPPLER

Decimal  9 = Tab key............ <tab>      <----Moves to next path.

Some notes related to the single-key commands--

1. Each single-key command is sent without any other key (like <lf>, etc).

2. Timing caution:  Since there is no return status after these

commands, the ATE remote controller program may have to delay for a

few tenths of a second to allow the PC controller to complete the

one-key command.  The amount of the delay is a function of the PC.

3. If the handshaking mentioned in note 2 is needed, the word commands should

be used exclusively.

HP 11759C/D Remote Control Documentation

- 28 -

Programming example for HP-IB:

The Channel Simulator can be controlled from BASIC using HP-IB.  The following

BASIC program (HP BASIC 4.0 or newer, running on a S200 or S300) illustrates

by example how this is done.

NOTE: Run this BASIC program before running HCHANSIM (11759C) or

HGHOST (11759D) on the PC.  If HCHANSIM or HGHOST is already running,

this BASIC program requires that the MAIN MENU is currently running on the PC.

10  !                             RE-SAVE  "SAMPLE_PGM"

20  !

30  ! This program demonstrates how to download a setup and enter manual

40  ! commands remotely to the Channel Simulator software from BASIC on a

50  ! HP 9836A using HP-IB connected to the Vectra.

80  !

90  ! TO RUN THIS PROGRAM:

100 !   o First, connect the HP-IB cable

120 !   o Second, Load and RUN this program.

130 !   o When the program says, start the HCHANSIM or HGHOST on the PC.

140 !   o Then press CONTINUE on the BASIC computer.

150 !

160 ! The program runs in two phases.

170 !

180 !   (1) The first phase downloads a Simulation Test setup from the

190 !       DATA statements at line 'Sim_setup' to the Channel Simulator.

200 !

210 !   (2) The second phase prompts the user to enter some remote commands

220 !       from the keyboard.  This program then downloads it to the remote

230 !       controlled PC, and handles any resulting status or error messages.

240 !

250 ! THE IMPORTANT PART of this routine is the last subprogram in this

260 ! listing named 'Send_a_cmd()'.  It sends one command to the PC

270 ! and handles any error messages by

280 ! printing them to the ATE controller's screen.

290 !

300 ! This was written to run on a HP 9836A, a series 200 computer, running

310 ! HP BASIC 4.0.  It should run on just about any version of HP "RMB" Basic

320 ! that can do the HP-IB I/O.

330 !

340 DIM Cmd$[80]           ! Cmd to send to the Channel Simulator is put here.

350 !

360 GOSUB Main_init        ! Init SelectCode for HP-IB port

370 !

380 DISP "Get HCHANSIM/HGHOST running on PC in main menu, then CONTINUE on this keybd:"

390 PAUSE

400 DISP

410 !

420 RESTORE Sim_setup      ! Set the READ statment to read a Rayleigh test setup

430 GOSUB Send_setup       ! Read each DATA statement, send the cmd to the PC.

440 !

450 GOSUB Indiv_cmd        ! Send indivual commands from the keyboard to the PC

HP 11759C/D Remote Control Documentation

- 29 -

460 DISP "Demo completed."

470 STOP                   ! Stop here, the demo has completed.

480 !

490 !

500 !

510 !!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

520 !!!!!!!          SUBROUTINES AND SUBPROGRAM FOLLOW    !!!!!!!!!!!!!!!!!!!!

530 !!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

540 !

550 Main_init:               ! !!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

560 ! Initialize the HP-IB select code variable 'Sc'

570 Sc=7                    ! The select code of the HP 98628A interface card.

580 Address=14              ! The HP-IB address of the PC running the

590 !                       ! Channel Simulator software.

590 Dev=Sc*100+Address      ! Device code used by OUTPUT and ENTER commands

600 !

720 RETURN                   ! End of 'main_init' subroutine, back to 'main'.

730    !

740    !

750 Sim_setup:      !        ! !!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

760     ! These data statements define word remote commands which are sent

770     ! from the HP 9836A (or equivalent ATE controller) to a PC which is

780     ! running the HP 11759C/D Channel Simulator software.

780     ! The second parameter is how many seconds to wait before sending

785     ! any more HP-IB commands.

790  DATA "*IDN?",0              ! Query the Channel Simulator Software's rev

800  DATA "PROG:NAME?",0         ! Query currently disp menu; should be MAIN

810  DATA "PROG:NAME SIMULATION",10! Choose to run Channel Simulation Test and

820  !                             ! must wait 10 seconds before outputting more

830  DATA "SMOD1:DEL 5E-6",0     !   Set path 1 delay to 5 microseconds.

840  DATA "SMOD1:DOPP:FREQ 10",0 !   Set path 1 Doppler to 10.0 Hz.

850  DATA "SMOD1:ATT 2",0        !   Set path 1 attenuation to 2 dB.

860  DATA "SMOD1:SPEC DOPP",0    !   Set path 1 to Doppler spectrum mode.

880  DATA "SMOD2:DELAY 10E-6",0  !   Set path 2 delay to 10 microseconds.

890  DATA "SMOD2:DOPP:FREQ 20",0 !   Set path 2 Doppler to 20.0 Hz.

900  DATA "SMOD2:ATT 4",0        !   Set path 2 attenuation to 4 dB.

910  DATA "SMOD2:SPEC DOPP",0    !   Set path 2 to Doppler spectrum mode.

930  DATA "SMOD3:DELAY 15E-6",0  !   Set path 3's delay to 15 microseconds.

940  DATA "SMOD3:DOPP:FREQ 41.7",0   !   Set path 3 Doppler to 41.7 Hz.

950  DATA "SMOD3:ATT 6",0        !   and set path 3's attenuation to 6 dB.

960  DATA "SMOD3:SPEC DOPP",0    !   Set path 3 to Doppler spectrum mode.

980  DATA "SMOD4:DEL 20E-6",0    !   Specify 20 microseconds delay for path 4.

990  DATA "SMOD4:DOPP:FREQ 40",0 !   Set path 4 Doppler to 40 Hz.

1000 DATA "SMOD4:ATT 8",0        !   Specify 8 dB attenuation for path 4.

1020 DATA "SMOD4:SPEC DOPP",0    !   Set path 4 to Doppler spectrum mode.

1040 DATA "SMOD5:DEL 25E-6",0    !   and set it's delay to 25 microseconds.

1050 DATA "SMOD5:DOPP:FREQ 50.0",0   !   Set path 5 Doppler to 50 Hz.

1060 DATA "SMOD5:ATT 10",0       !   and path 5's attenuation to 10 dB.

1080 DATA "SMOD5:SPEC DOPP",0    !   Set path 5 to static Doppler mode.

1100 DATA "SMOD6:DEL 30E-6",0    !   and set path 6's delay to 30 microseconds.

1110 DATA "SMOD6:DOPP:FREQ 41.7",0   !   Set path 5 Doppler to 41.7 Hz.

1120 DATA "SMOD6:ATT 12",0       !   and path 6's attenuation to 12 dB.

HP 11759C/D Remote Control Documentation

- 30 -

1140 DATA "SMOD6:SPEC DOPP",0    !   Set path 6 to static Doppler mode.

1150 DATA "",0                   ! This blank string tells output loop to stop

1160   !

1170   !

1180 Send_setup:               ! !!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

1190  ! Read DATA statements (set by previous RESTORE cmd) and send each string

1200  ! out to the PC that's running the Channel Simulator software.

1210  REPEAT                       ! Read each command that's part of the

1220    READ Cmd$,Wait_time           ! setup to be downloaded from DATA.

1240    IF Cmd$<>"" THEN              ! If this cmd is not the end-of-list "",

1250      CALL Send_a_cmd(Sc,Dev,Cmd$,Wait_time) ! go ahead & send to the PC.

1260    END IF                        !   & handle error messages.

1280  UNTIL Cmd$=""                ! Keep sending DATA cmds till @end

1285  RETURN

1290  !

1295  !

1300 Indiv_cmd:                 ! !!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

1310  ! Keep accepting one command from the keyboard, and sending it to the

1320  ! Channel simulator, until the user presses RETURN to stop.

1330  REPEAT

1335    LINPUT "Enter the remote command for the Channel Simulator: ",Cmd$

1340    Wait_time=0                     ! Assume no waiting

1345    INPUT "Enter the wait time before sending more HP-IB commands:",Wait_time

1350    IF Cmd$<>"" THEN                ! If user did not just press RETURN

1360      CALL Send_a_cmd(Sc,Dev,Cmd$,Wait_time) ! cmd entered; send it to PC

1370    END IF                          !   and handle error messages.

1380  UNTIL Cmd$=""                     ! Keep asking user for more cmds

1390  RETURN                            ! until no more entered, then return

1400  !

1410  !

1420  END                               ! End of main & subroutines, begin

1430                                    ! defining the subprograms (only 1).

1440  !

1450  !

1460  !!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

1470  !!!!!!!                 SUBPROGRAM   DEFINED          !!!!!!!!!!!!!!!!!!!!

1480  !!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

1490  !

1500 Send_a_cmd:SUB Send_a_cmd(Sc,Dev,Cmd$,Wait_time) ! !!!!!!!!!!!!!!!!!!!!!!!!

1510 ! Send a command to the remote controlled PC.  The 'Sc' param is the select

1515 ! code of the HP-IB card that is installed in the HP 9836A (or equivalent)

1520 ! computer.  The 'Dev' param contains the the select code and 'Dev'

1530 ! contains the HP-IB bus address of the PC running the Channel Simulator

1535 ! software.  The 'Cmd$' is the command to sent to the PC.

1540 ! The 'Wait_time' is the number of seconds to wait after sending the

1545 ! command before any other HP-IB commands can be sent.

1550 !

1555 ! ALGORITHM:

1560 !  Clear the input buffer--

1570 !    o First, clear the Channel Simulator software's error queue.

1590 !  Preprocess the command--

1600 !    o The command is trimmed of leading & trailing spaces.

HP 11759C/D Remote Control Documentation

- 31 -

1610 !    o If there was no entered command, then exit without doing anything

1620 !      else.

1630 !    o Print to this ATE controller computer's screen the cmd to be sent

1640 !      out.

1660 !  Send the Command to the PC--

1690 !    o The cmd is sent as a word command with a trailing <lf>.

1690 !    o If command was a query, enter the response and display it.

1690 !    o Wait for 'Wait_time' seconds

1710 !  Check the error queue for errors (if any)--

1750 !    o Go into a loop & keep reading &

1770 !      printing any messages, until the +0,"No error" message

1780 !      is read.  The expected return traffic is one of:

1790 !           +0,"No error"<lf>         or       <ErrMsg><lf>

1800 !  Then exit back to the caller.

1810 !

1820 DIM A$[2000]                ! Input lines are read back into this string.

1830 !

1840 ! *** Clear the Channel Simulator software's error queue

1900 OUTPUT Dev;"*CLS"           ! Clears the error queue

1910 !

1920 ! *** Preprocess the command.

1930 Cmd$=TRIM$(Cmd$)            ! Remove any leading & trailing spaces from cmd

1940 IF LEN(Cmd$)=0 THEN SUBEXIT ! If the cmd being sent is nonexistent, exit.

1950 !

1990 PRINT Cmd$                  ! print the cmd which will be sent to the PC.

2010 !

2020 !

2030 ! *** Send Command to the PC (which is running the Channel Simulator sw)

2040 OUTPUT Dev;Cmd$             ! output it with a trailing <lf> char.

2050 IF POS(Cmd$,"?") THEN       ! If the command was a query

2055   ON TIMEOUT Sc,5 GOTO Wait_here  ! Set timeout in case query was bad

2060   ENTER Dev;A$              !   Read in the query response

2070   PRINT A$                  !   Print the query response

2075 END IF                      !

2080 !

2085 Wait_here: !

2090   OFF TIMEOUT               ! Turn off any timeouts

2095 WAIT Wait_time              ! Wait if required

2100 !

2105 !

2110 ! *** Check the error queue for errors (if any).

2200 REPEAT                      ! Repeatedly query the error queue

2210   OUTPUT Dev;"SYST:ERR?"    ! Get next msg from PC, <lf> terminated

2220   ENTER Dev;A$              ! Get next msg from PC, <lf> terminated

2230   PRINT A$                  ! Print the error message.

2240 UNTIL A$="+0,""No error"""  ! Keep reading until queue is empty.

2250 SUBEND                      ! Finally, exit back to calling routine