You won't believe it, but the LptPort.dll is made to access the parallel port(s).

All functions are (inline-) assembler calls , the LPT addresses are read from

a)  Win9x         :    BIOS Data Area ( The 4 possible entries ) ,

b)  NT,2K,XP  :   Registry .

The default for LPT1 is 0x378 , if the actuell address differs or you want to use another LPT-port it must be set by 'SelectPort' ( with the address as argument). The function 'MyLptAdr' returns '0' if the port with the choosen number isn't installed.

This DLL does  NOT work on NT..XP without the below described preparations , because these systems prevent direct port access.

Work around :

Use the excellent (and good documented) utility UserPort from

http://www.embeddedtronics.com/design&ideas.html

This utility 'unlocks' selected ports for user access (for all programs !).

It works really fine !

Dr.Detlef Kersten